| ASTRAQ ET-M |  |
| --- | --- |

18 kg-class collaborative robotic massage and therapy platform for wellness, physiotherapy and recovery applications.

| ET-M Robotic Massage System | Key Configuration Series; ET-M CR18-18/1.0. Robot architecture; 6-axis collaborative robotic therapy arm. Payload; 18 kg. Reach; 1,062 mm. Repeatability; ±0.03 mm. Typical power; 600 W. Noise; ≤ 70 dB(A). Protection rating; IP54. Programming; Direct teaching control and graphical interface. Application focus; Robotic massage, rehab assistance, wellness and sports recovery. SYSTEM POSITIONING ET-M combines a soft medical-wellness exterior with a class collaborative robot platform. It is designed for controlled, repeatable and configurable therapy paths in professional treatment spaces under operator supervision. |
| --- | --- |

CORE ADVANTAGES

| Force-aware therapy Built-in joint sensing supports compliant and safe interaction. | High payload margin 18 kg payload supports robust therapy end-effectors. | Practical reach 1,062 mm working reach for table-side treatment coverage. | Easy workflow setup Direct teaching and graphical programming reduce deployment time. | Clinic-ready design Quiet, smooth and premium appearance for wellness environments. |
| --- | --- | --- | --- | --- |

Technical Parameters

| ROBOT ARM Model / Series; ET-M CR18-18/1.0. Robot type; 6-axis collaborative robotic therapy arm. Payload; 18 kg. Reach; 1,062 mm. Robot weight; About 38 kg. Degrees of freedom; 6. MTBF; > 80,000 h. Power supply; 48 VDC. Typical power; 600 W. PERFORMANCE AND MOTION Repeatability; ±0.03 mm. Force sensing; Force x-y-z: 0.1 N; Torque x-y-z: 0.02 Nm. Cartesian stiffness; 0-18,000 N/m; 0-2,500 Nm/rad. Joint working range; Axis 1 ±360°; Axis 2 ±360°; Axis 3 ±168°; Axis 4 ±360°; Axis 5 ±360°; Axis 6 ±360°. Max joint speed; Axis 1 163°/s; Axis 2 163°/s; Axis 3 135°/s; Axis 4 155°/s; Axis 5 199°/s; Axis 6 228°/s. Max tool-end speed; ≤ 6.0 m/s. | SAFETY AND ENVIRONMENT Safety functions; More than 21 adjustable safety features including collision detection, virtual walls and collaboration mode. Safety references; EN ISO 13849-1; EN ISO 10218-1 / PL d, Cat. 3; ISO 15066; CE, KCs and EAC marking requirements. IP rating; IP54. ISO cleanroom class; Class 5. Noise; ≤ 70 dB(A). Temperature; 0°C to 50°C. Humidity; ≤ 93% RH, non-condensing. Installation orientation; At any angle. CONTROLLER AND INTERFACE Controller; Built-in controller option for CR18-class architecture. Operator interface; Notebook / PAD / drag interactive module. Safety devices; Handheld enable and handheld emergency stop. Communication; TCP/IP 1000Mbit, Modbus TCP, Profinet, Ethernet/IP, DeviceNet, CC-Link, CC-Link IE Field Basic. Tool I/O; 2 digital outputs, 2 digital inputs, 2 analog inputs. Tool communication; RS485; alternatively two analog input pins. Tool I/O power; 12 V / 24 V, 1 A rated. Base output power; 24 V, 1.5 A. Teach pendant; xPad2, 10.1-inch LCD, 1920 x 1200, IP54. |
| --- | --- |

THERAPY APPLICATION PACKAGE

| Massage workflow Configurable back, shoulder and limb massage paths for treatment-bed scenarios. | End-effector options Massage, percussion, pressure or soft-tissue therapy modules can be configured. | Human-machine safety Collaborative safety setup supports shared therapist, client and robot workspaces. | Future integration Open interfaces can support treatment UI, scheduling and therapy-data platforms. |
| --- | --- | --- | --- |

Note: Final specifications may be refined for the ET-M therapy end-effector, software interface and clinical workflow package.