| ASTRAQ | DPM1 6x4 Diesel Prime Mover Formal product specification |
| --- | --- |

DPM1 6x4 Diesel Prime Mover

| KEY DIMENSIONS Application; 6x4 prime mover / semi-trailer tractor. Length (mm); 7025. Width (mm); 2490. Height (mm) unloaded; 3317. Wheelbase (mm); 3200 + 1400. GVM (kg); 25,000. GCM (kg); 90,000. Max speed (km/h); 100. Fuel tank (L); 700 + 240. Fifth-wheel height (mm); 1260 unloaded. | FEATURES • 6x4 diesel prime mover with one front steering axle and tandem rear drive axles. • Right-hand drive cab configured for Australian prime mover applications. • MC13.54-61 12.42L diesel engine: 400 kW / 1800 rpm and 2500 Nm / 950-1400 rpm. • ZF 12TX2621TD 12-speed automated transmission with overdrive top gear. • 25,000 kg GVM and 90,000 kg GCM. • Tandem rear drive axles with inter-wheel and inter-axle differential locks. • Dual-circuit compressed-air braking with EBS, ESC and hill-start assist. • ACC, AEBS, LDW and reversing camera included for driver assistance. |
| --- | --- |

| ASTRAQ | DPM1 6x4 Diesel Prime Mover Vehicle parameters |
| --- | --- |

| WEIGHTS Kerb weight (kg); 8,800. Kerb axle load - front (kg); 4,820. Kerb axle load - rear (kg); 3,980. GVM (kg); 25,000. GCM (kg); 90,000. Design axle load - front (kg); 7,000. Design axle load - rear (kg); 18,000. CAPABILITY Max speed (km/h); 100. Minimum turning diameter (m); 14. Max gradeability (%); ≥34. Max parking grade (%); ≥20. Fuel tank capacity (L); 700 + 240. CABIN / COMFORT Cab model; DPM1 RHD prime mover cab. Passenger capacity; 2. Cab suspension; Four-point air suspension. Driver seat; Air suspension seat. Cab tilting; Electric + manual hydraulic. Multimedia; MP5. Air conditioning; Semi-automatic. Remote key; 1 key. | DIMENSIONS Length (mm); 7025. Width (mm); 2490. Height (mm) unloaded; 3317. Wheelbase (mm); 3200 + 1400. Wheel track - front (mm); 2041. Wheel track - rear (mm); 1842. Front overhang (mm); 1475. Rear overhang (mm); 950. Approach angle; 19°. Departure angle; 22°. Fifth-wheel height (mm); 1260 unloaded. Front / rear fitting radius (mm); 2280 / 1961. PRIME MOVER INTERFACE Application interface; Rear fifth-wheel coupling. Semi-trailer connection; Included. Axle layout; 1 front steering + 2 rear drive axles. |
| --- | --- |

| ASTRAQ | DPM1 6x4 Diesel Prime Mover Powertrain and chassis |
| --- | --- |

| ENGINE Model; MC13.54-61. Displacement (L); 12.42. Power rating; 400 kW / 1800 rpm. Max torque; 2500 Nm / 950-1400 rpm. Application; Heavy-duty diesel prime mover. TRANSMISSION AND CLUTCH Transmission model; FAST. Forward ratios; 16.688, 12.924, 9.926, 7.688, 5.895, 4.656, 3.655, 2.831, 2.174, 1.684, 1.291, 1.00. Reverse ratios; 15.537, 12.033. Clutch model; 430 push-type diaphragm spring clutch. Clutch operation; Electronically controlled pneumatic power assist. | AXLES AND SUSPENSION Front axle model; VPD71DS. Driving axle model; MCY13BES. Final drive ratio; 3.08 / 3.36 / 3.70. Differential lock; Inter-wheel and inter-axle differential lock. Suspension model; Non-independent suspension. Front suspension; Parabolic springs, 3 leaves. Rear suspension; Air suspension, 8 air springs. TYRES AND FRAME Rim; Front 22.5 x 9.0 aluminium alloy; rear 22.5 x 8.25 aluminium alloy. Tyres; Front 295/80R22.5; rear 11R22.5. Frame model; Trapezoidal frame with wide front and narrow rear. Main beam section (mm); Slot type 270 x 85 x 8. Frame width (mm); Front 942; rear 762. |
| --- | --- |

| ASTRAQ | DPM1 6x4 Diesel Prime Mover Steering, braking, safety and electrics |
| --- | --- |

| STEERING Model; Recirculating ball hydraulic power steering. Steering ratio; 17-20:1. Max pump pressure (kPa); 20,000. Working pressure (kPa); 1,250. Steering way; Steering wheel. BRAKING / SAFETY Service brake; Dual-circuit compressed-air brake. Brake power assist type; EBS + ESC + HSA. Parking brake type; Spring stored energy air cut brake. Circuit type; Dual. Driver assistance; ACC + AEBS + LDW. Reverse camera; Included. | ELECTRICS / COMFORT Operating voltage; 24 V. Generator power; 3 kW. Batteries; 2 x 12 V / 240 Ah. Multimedia; MP5. Air conditioning; Semi-automatic. Remote key; 1 key. PRODUCT POSITIONING • DPM1 is configured as a 6x4 diesel prime mover for semi-trailer operation. • The rear fifth-wheel coupling distinguishes DPM1 from a rigid cab chassis. • The specification emphasises GCM, fifth-wheel height, tandem drive axles, air suspension, long-haul braking and driver-assistance functions. • This document presents DPM1 as a product specification rather than a comparison or target configuration sheet. |
| --- | --- |

| ASTRAQ | DPM1 6x4 Diesel Prime Mover Chassis structure layout |
| --- | --- |

STRUCTURE DRAWING

DPM1 6x4 prime mover chassis layout - side view and top view

| LAYOUT HIGHLIGHTS Axle configuration; 1 front steering axle + 2 rear drive axles. Wheelbase (mm); 3200 + 1400. Application interface; Rear fifth-wheel coupling. Frame arrangement; Trapezoidal ladder frame. Rear suspension; Air suspension, 8 air springs. Fuel system; 700 + 240 L. | PACKAGING NOTES • Side-view layout supports fifth-wheel, axle-spacing and rear overhang review. • Top-view layout supports frame, driveline and component-packaging review. • Chassis configuration is arranged for semi-trailer operation rather than direct rigid-body mounting. |
| --- | --- |