(Reference Photo)

EXTERNAL DIMENSIONS

| Length (mm) | 8615 |
| --- | --- |
| Width (mm) | 2518* |
| Height (mm) (empty) | 3120 |
| Wheel base (mm) | 1800+3000+1350 |
| Wheel track (mm) | Front 2030/2030; Rear 1850/1850 |
| Front overhang (mm) | 1490 |
| GVM (Kg) | 35000 |
| Max speed (km/h) | 100 |
| Max climb gradient (%) | Theoretical 54% |
| Fuel tank (L) | 250L |

FEATURES

•  Cummins X11EVIE360 10.5L engine; 265 kW / 1900 rpm and 1800 Nm / 1000-1400 rpm.

•  F12JZ20A 12-speed transmission with 0.78 overdrive top gear.

•  Right-hand drive flat-roof short semi-sleeper cab for mixer applications.

•  Four-point air suspension cab and air suspension driver seat.

•  7.5t disc front axle and 10t drum rear axle with dual differential locks.

•  Four-airbag rear suspension designed for mixer platform stability.

•  EBS + ESC, AEBS, FCWS, LDWS and reversing camera included.

•  Right-side water tank reserved space: 1335 mm; customer target: 2000 mm.

WEIGHTS

| GVM (kg) | 35000 |
| --- | --- |
| Front axle rating | 7.5t + 7.5t |
| Rear axle rating | 10t + 10t |
| GCM (kg) | To be confirmed |

CAPABILITY

| Max speed (km/h) | 100 |
| --- | --- |
| Max climb gradient (%) | 54 |
| Fuel tank (L) | 250 |
| Turning diameter | <20 |

ENGINE

| Model | Cummins X11EVIE360 |
| --- | --- |
| Displacement (L) | 10.5 |
| Power rating | 265 kW / 1900 rpm |
| Max torque | 1800 Nm / 1000-1400 rpm |

TRANSMISSION

| Model | F12JZ20A |
| --- | --- |
| Forward ratios | 12.26, 9.56, 7.36, 5.73, 4.52, 3.52, 2.71, 2.11, 1.63, 1.27, 1.00, 0.78 |

FRAME

| Model | Ladder framework |
| --- | --- |
| Main beam | 1000-850 x 300 x 80 x (8+4) |
| Reserved water tank space | Right side 1335 mm |

FRONT AXLE

| Model | 7.5t disc front axle |
| --- | --- |
| Type | Mercedes type front axle |
| Wheel track | 2030 / 2030 mm |

DRIVING AXLE

| Model | 10t drum oil-lubricated 440 axle |
| --- | --- |
| Differential lock | Interwheel / interaxle diff. lock |
| Ratio | 4.44 |
| Wheel track | 1850 / 1850 mm |

SUSPENSION

| Front | Few-leaf spring 3/3 |
| --- | --- |
| Rear | Four airbags, 2+ - / 2+ - |
| Cab suspension | Four-point air suspension |

CABIN

| Model | Flat-roof short semi-sleeper |
| --- | --- |
| Steering | Right-hand drive |
| Cab tilting | Electric + manual hydraulic |
| Seat | Air suspension seat |

BRAKING / SAFETY

| EBS + ESC | Included |
| --- | --- |
| AEBS | Included |
| FCWS | Included |
| LDWS | Included |
| Reverse camera | Included |

ELECTRICS / COMFORT

| Battery | 180 Ah low-temperature |
| --- | --- |
| Remote key | 2 key |
| Multimedia | MP5 |
| Air conditioning | Semi-automatic |

Preliminary chassis layout drawing (non-final status)