| ASTRAQ | ASTRAQ Electric Light Truck 4x2 Battery-Electric Urban Logistics Truck |
| --- | --- |

Key Configuration

| Model positioning | 4x2 battery-electric light-duty box truck for urban and metro distribution |
| --- | --- |
| Overall dimensions (mm) | 5,995 x 2,200 x 3,150 (box-body configuration) |
| Wheelbase (mm) | 3,400 |
| GVM | 4,495 kg |
| Body type | Dry van / box body; refrigerated body available |
| Traction battery | LFP / blade battery, liquid-cooled pack |
| Battery capacity (kWh) | 100 kWh standard; 132 kWh long-range option |
| Motor power | 70 kW rated / 150 kW peak |
| Driving range | 220-300 km typical urban operation, depending on load and body type |
| Charging method | CCS2; AC 11 kW, DC fast charge up to 70 kW |

astraq.com.au  |  Electric Light Truck Specification

| ASTRAQ | ASTRAQ Electric Light Truck Technical Specification |
| --- | --- |

Vehicle & Chassis

| Drive configuration | 4x2 |
| --- | --- |
| Vehicle category | 4.5t electric light-duty truck |
| Cab | Forward-control EV cab, white exterior, aerodynamic front styling |
| Body | Integrated box body for city delivery applications |
| Frame | Ladder frame with box-body mounting provision |
| Front axle capacity | 2,600 kg |
| Rear axle capacity | 4,500 kg |
| Suspension | Front independent / coil spring; rear leaf spring with shock absorbers |
| Tyres | 17.5-inch commercial tyres; 205/75R17.5 reference size |
| Turning circle | Approx. 12.8 m for 3,400 mm wheelbase configuration |

Electric Drive, Battery & Charging

| Motor | Permanent-magnet synchronous motor |
| --- | --- |
| Peak torque | 430 Nm peak / 250 Nm continuous |
| Transmission | Single-speed reduction gearbox; no gear changes |
| Battery chemistry | LFP / blade battery |
| Battery capacity | 100 kWh standard; 132 kWh option |
| Battery cooling | Liquid-cooled battery thermal management |
| Charging standard | CCS2 |
| AC charging | Up to 11 kW |
| DC fast charging | Up to 70 kW |
| Auxiliary power | 12V / 24V electrical system with body-builder support |

Safety, Intelligence & Comfort

| Active safety | AEB / Active Brake Assist, LDWS, ESP/ESC, ABS + EBD |
| --- | --- |
| Urban safety | Acoustic Vehicle Alerting System (AVAS), reverse warning buzzer |
| Lighting | LED headlamps, LED taillamps and daytime running lamps |
| Braking | Hydraulic dual-circuit disc brakes with regenerative braking |
| Parking brake | Electric park brake |
| Cab safety | ECE-R29 cab strength concept; driver and passenger SRS airbags |
| Driver interface | Digital display with speed, battery percentage, range and temperature |
| Comfort | Air conditioning / heater / demister; electric windows; central locking |

Designed for zero-emission urban logistics, quiet delivery operation and practical city access.